Freebie from The Embroidery Garden

http://members.fortunecity.com/reen5/index.htm

lmom.zip contains the 4x4 & 5x7 design

- Appliqué Position. A running stitch will be sewn showing where the heart will sew out.

- Appliqué Material. Before sewing out step #2, iron Wunder Under, Heat N Bond, or whatever fusible you like to use, to the back of the fabric you’re using for the heart appliqué. Sew step #2. Remove the hoop from the machine, but DO NOT unhoop your fabric. Carefully trim around the heart shape very close to the running stitches. Then carefully iron the fusible backed fabric down using the tip of the iron if necessary.

- Satin stitch outline of the heart

- Lettering